5MP 15~50MM Global Shutter Camera Module

- Video Encoding:H.265/H.264/MJPEG

- Video resolution:2448*2048;

- Global shutter max to 5MP/60FPS

- Integrated neural network acceleration engine, computing power up to 2.5Tops INT8

- Dual ARM Cortex A55@1.2GHz processor, 32KB I-Cache, 32KB D-Cache ,256KB L3 cache

- USB3.0 video output,standard yuv protocol

- DC auto iris and CVBS can be optional ,need add one more expand board

- Default dual stream , main stream and sub stream

- Support AI super noise reduction, 3D noise reduction, 2D noise reduction, image edge enhancement, defogging, dynamic contrast enhancement, etc.

- Support automatic restart function after power failure/accidental failure;

Datasheet:

| Order Model:M568H19D_L21_10X |  |  |  |
| --- | --- | --- | --- |
| System |  |  |  |
| CPU Model | HI3519DV500 Dual ARM Cortex A55@1000MHz |  |  |
| CPU Ability | 2.5 Tops INT8 |  |  |
| Flash | 256Mbit SPI Nor Flash |  |  |
| DDR | 8Gbit DDR4 (2x 4Gbit) |  |  |
| Image |  |  |  |
| Sensor Board | SONY IMX568 1/1.8″Global Shutter Sensor |  |  |
| Video Resolution | 2488*2048 /5MP |  |  |
| Video Compression | H.265/H.264/MJPEG |  |  |
| Video Frame | Main stream:2488*2048/2048*1520/ 1920*1080 1-60fps Sub stream: 1280*720 /704*576/ 640*480 352*288 1-60fps |  |  |
| Lens | 15~50MM(Japan TAMRON brand) |  |  |
| Aperture | F1.4～F1.6 |  |  |
| Field angle |  | Wide | Tele |
|  | H | 26.3° | 8.1° |
|  | V | 20° | 6.2° |
| S/N Ratio | ≥52dB |  |  |
| 3D Noise | Support |  |  |
| Optical zoom | 4X |  |  |
| ICR Switch | Auto ICR full color switch to B/W |  |  |
| T-WDR | 120dB |  |  |
| BLC | Support |  |  |
| Focus control | Auto /Manual |  |  |
| Electronic shutter | 1/25—1/10000s |  |  |
| Min Illumination | Color 0.001 Lux@F1.6 |  |  |
| Audio |  |  |  |
| Audio Interface | 2ch audio in(1ch Line In &1ch MIC In) 1ch audio out,support voice intercom |  |  |
| Audio compression | G.711A、G.711U |  |  |
| Image |  |  |  |
| AE | support |  |  |
| AGC | Support |  |  |
| AWB | Support |  |  |
| 3D | Support |  |  |
| Privacy Mask | Up to set 4 Privacy mask area |  |  |
| Interesting area code | Support |  |  |
| Off network record video | Support |  |  |
| Motion detect | Up to set 4 motion detect area |  |  |
| Day&Night Switch | Built-in intelligent algorithm to auto control IR CUT switch,color and white&night auto switch |  |  |
| Video adjust | Adjustable image contrast, brightness, saturation, sharpness and image mirroring, flipping, fog penetration, strong light suppression, backlight compensation, noise reduction level, digital wide dynamic, etc. |  |  |
| Corridor mode | support |  |  |
| Remote manager | System setting/Video stream management/Account management/Network management |  |  |
| Alarm Detect | E-mail alarm,PC client alarm,Motion detect alarm |  |  |
| Function Interface | 1CH Reset |  |  |
|  | 1CH RS485(Optional) |  |  |
|  | 1CH alarm in(standard),1CH alarm out（optional,need add one board） |  |  |
|  | SD/TF card storage，max 1T storage（optional,need add one board） |  |  |
| OSD | OSD place ,color ,words can be config |  |  |
| Network |  |  |  |
| Ethernet Interface | 1PCS RJ45 100M |  |  |
| Protocol | TCP/IP、UDP、RTP、RTSP、RTCP、RTMP、HTTP、DNS、DDNS、DHCP、NTP、PPPOE、SMTP、UPNP; |  |  |
| ONVIF | ONVIF Profile S |  |  |
| General Info. |  |  |  |
| Power Supply | DC12V(±10%) |  |  |
| Power Consumption | TBD |  |  |
| Working temperature | -30°~+70° |  |  |
| Working humidity | 10%~90% |  |  |

Pin definition :

1.AF Board (Zoom/Focus/PTZ)

2.IPC camera （sensor+cpu board）