400W Starlight Class_20X Network Movement（400W星光级_20X网络机芯）

Product characteristics

- Video resolution：2688*1520/60fps ；Video encoding:H.265/H.264/MJPEG

- Excellent autofocus algorithm

- Integrated neural network acceleration engine, computing power up to 2.5Tops INT8

- Supports Neon accelerate and integrates FPU processing units

- Dual ARM Cortex A55@1.2GHz processor, 32KB I-Cache, 32KB D-Cache ,256KB L3 cache

- Support AI super noise reduction, 3D noise reduction, 2D noise reduction, image edge enhancement, defogging, dynamic contrast enhancement, etc.

- Support to remove fixed pattern noise (FPN)

- Supports OSD character overlay and BMP image overlay, with adjustable overlay position

- Supports ONVIF protocol and national standard GB/T28181 protocol;

- Adopting H.264 H.265 video encoding, users can choose the stream and adjust the resolution, frame rate, and video quality;

- Support motion detection alarm and client alarm;

- Support bad pixel correction and lens shadow correction;

- Supports motion detection/occlusion of images, with the ability to set up 4 occlusion areas and 4 detection areas;

- Support network time synchronization and RTC power-off timing function;

- Support remote real-time monitoring, network user management, and network time synchronization;

Specification parameters

| Order Model |  | M664H19D5_S_L21_20X |  | M664H19D5_E_L21_20X |
| --- | --- | --- | --- | --- |
| System |  |  |  |  |
| CPU Model |  | HI3519DV500 Dual ARM Cortex A55@1000MHz |  |  |
| CPU ability |  | 2.5 Tops |  |  |
| Flash |  | 256Mbit SPI Nor Flash |  | 8GB eMMC |
| DDR |  | 8Gbit DDR4 (2 X 4Gbit) |  | 8Gbit DDR4 (2 X 4Gbit) |
| Image |  |  |  |  |
| Sensor Board |  |  | SONY IMX664，1/1.8”Progressive Scan CMOS Sensor |  |  |
| T-WDR |  |  | ＞120db |  |  |
| Video Resolution |  |  | 2688(H)x1520(V) |  |  |
| Lens |  |  | 6.7～134.5mm±5％ |  |  |
| Aperture |  |  | F1.5(Wide)～F3.4(Tele)±5％ |  |  |
| Field angle |  |  |  | Wide | Tele |
|  |  |  | 对角(D) | 70° | 3.76° |
|  |  |  | 水平(H) | 59.8° | 3.0° |
|  |  |  | 垂直(V) | 44.4° | 2.24° |
| S/N Ratio |  |  | ≥54db |  |  |
| 3D Noise |  |  | Support |  |  |
| Optical zoom |  |  | 20X |  |  |
| ICR Switch |  |  | Auto ICR full color switch to B/W |  |  |
| Wide Dynamic |  |  | Support |  |  |
| BLC |  |  | Support |  |  |
| white balance |  |  | Automatic/Indoor/Outdoor |  |  |
| Focus control |  |  | Auto /Manual |  |  |
| Electronic shutter |  |  | 1/25—1/10000s |  |  |
| Min Illumination |  |  | Color 0.001 Lux@(F1.6,AGC ON) |  |  |
| Audio video encoding |  |  |  |  |  |
| Video Encoding |  |  | H.264 H.265 MJPEG编码 |  |  |
| Video Frame |  | Main stream | 2688*1520 2560*1440 2048*1520 1920*1080 1-60fps |  |  |
|  |  | Sub stream | 1280*720 704*576 640*480 352*288 1-60fps |  |  |
| Video Bit rate |  |  | 32Kbps~80Mbps continuously adjustable, supporting CBR/VBR |  |  |
| Subtitle Overlay |  |  | Support adjustable stacking position of channel name, date and time |  |  |
| Audio Encoding |  |  | G.711A / G.711U |  |  |
| Audio/Intercom Input |  |  | 2ch audio in(1ch Line In &1ch MIC In) |  |  |
| Audio Output |  |  | 1ch audio out |  |  |
| Storage Interface |  |  | Support SD/TF card storage,max 1T storage(need add one board) |  |  |
| Ethernet Interface |  |  | 10/100M network adaptive, RJ45 adapter |  |  |
| Data Transmission and Storage |  |  |  |  |  |
| Data Storage |  |  | Video and image files |  |  |
| Storage Mechanism |  |  | Manual, automatic (cyclic, timed, alarm switch, motion detection, occlusion detection) |  |  |
| Alarm Data Transmission |  |  | Email, browser, management software |  |  |
| Protocol |  |  | TCP/IP、UDP、RTP、RTSP、RTMP、RTCP、HTTP、DNS、DDNS、DHCP、NTP、PPPOE、SMTP、UPNP |  |  |
| Wireless Protocol Support (-W) |  |  | 802.11b/g/n, Support WEP, WPA, WPA2 encryption protocols |  |  |
| Client Browsing |  |  | Support embedded Web Server, support concurrent access by 8 users (MAX) |  |  |
| Access Protocol |  |  | ONVIF, GBT28181 B protocol |  |  |
| General Info |  |  |  |  |  |
| Power Supply |  |  | DC12V power supply/POE power supply (need add POE expansion board) |  |  |
| working environment |  |  | Temperature:-20°~+70° ; humidity:0% -90% |  |  |

Pin definition :